TEM Consulting

# Scanning Positioner

# Data Sheet

| ## Physical Dimensions |  |  |
| --- | --- | --- |
| Movement Range (X-Y-Z) | 45 x 45 x 47 cm (17.75 x 17.75 x 18.5 in) | Maximum movement protected by limit switches which can be adjusted to smaller dimensions if needed |
| Dimensions | 91 x 91 x 96.5 cm (36 x 36 x 38 in) |  |
| Recommended Operating Area | 91 x 124 x 140 cm (36 x 49 x 55 in) |  |
| ## Movement Specifications |  |  |
| ### Accuracy | ±0.0833% (±0.083 cm/meter / 0.010 in/ft) |  |
| Time per step (typical) | 0.12 sec + measurement time | Based on recommended movement speed to minimize vibration |
| Time per step (minimum) | 0.02 sec + measurement time |  |
| Total Scan Time (typical) | 8:50 min. | Based on 5 cm x 5 cm scan in 2 mm steps (676 measurements) with a 20 mS instrument read time. |
| Min. Step Size | 0.004 mm |  |
| Max. Speed | 6.1 cm/s (2.4 in/s) |  |
| ## Computer Requirements |  |  |
| ### Interfaces Available | Parallel Port GPIB USB PCI PXI |  |
| Operating Software | LabVIEW TILE | LabVIEW may be ordered separately, if required. |
| Processor (min.) | Pentium 200 MHz |  |
| RAM (min.) | 64 MB |  |
| Screen Resolution (min.) | 800 x 600 pixels |  |
| Operating System | Windows XP/2000/NT 4.0 SP6a/ME/98 |  |
| Disk Space | 25 MB |  |
| ## Power |  |  |
| ### Line Voltage | 115 VAC |  |
| Line Frequency | 60 Hz |  |